INDIA SPACE LAB

भारत अंतरिक्ष प्रयोगशाला

CanSat Ground Control Software (GCS)

Project Report

Real-Time Telemetry Monitoring and Mission Operations Software

| Field | Details |
| --- | --- |
| Student Name | Boda Shanmukha Datta |
| Institution | National Institute of Technology (NIT) Jalandhar |
| ISL Enrolment Number | ISL-177115 |
| Email | bodasd.ic.24@nitj.ac.in |
| Course Domain | Aerospace Engineering / Embedded Systems / Avionics / Ground Systems |
| Assignment Type | Design and Development Assignment |
| Project Category | Real-Time Telemetry Monitoring and Mission Operations Software |

Submitted to: India Space Lab, Bharat Antariksh Prayogshala

# Table of Contents

# 1. Assignment Overview

This report documents the design and development of a single-page CanSat Ground Control Software (GCS), built to monitor telemetry in real time, visualize mission parameters, display live GPS tracking, handle mission-critical controls, and simulate aerospace mission operations. The software was developed as a modular, readable, mission-oriented web application, following the structure and priorities used by professional ground-segment aerospace systems.

The deliverable is a fully functional, browser-based GCS (HTML5, CSS3, JavaScript) that receives telemetry over the Web Serial API from a microcontroller (WeGyanik Kit) running a dummy telemetry generator, and renders that telemetry across six coordinated display sections in a single page.

# 2. System Architecture

The application is organised as a set of independent JavaScript modules that each own one concern, and communicate through a small central telemetry store. This keeps the interface single-page while remaining easy to extend.

| Module | Responsibility |
| --- | --- |
| telemetry.js | Parses incoming CSV telemetry lines into structured packet objects; keeps the in-memory packet history used for CSV export. |
| errorcodes.js | Implements the 4-digit live fault monitoring logic and drives the 7-segment error display. |
| charts.js | Owns five Chart.js real-time line graphs (altitude, pressure, temperature, descent rate, battery voltage). |
| map.js | Owns the Leaflet.js / OpenStreetMap live tracking map, marker and path polyline. |
| orientation.js | Owns the Three.js 3D attitude visualization driven by roll / pitch / yaw. |
| video.js | Owns MediaDevices-based live video streaming, camera selection and stream status. |
| serial.js | Owns the Web Serial API hardware link and a built-in demo telemetry simulator. |
| datamanager.js | Fan-outs each parsed packet to every visualization module; owns CSV export, graph export and reset/storage. |
| missioncontrol.js | Owns the three mission-critical command buttons, confirmation prompts and the command execution log. |
| app.js | Boots all modules, drives the mission/UTC clocks, wires Sync PC Time / Reset Packet buttons. |

# 3. Interface Layout

The dashboard is built as a single HTML page using CSS Grid for the outer layout and Flexbox within each panel, satisfying the requirement for a single-page, professional aerospace-style operator dashboard.

- Top control bar — mission link status, mission/UTC clock, and all operator controls

- Left column — Container telemetry, Payload telemetry, and the 4-digit Error Code module

- Centre columns — Live Tracking Map and 3D Orientation visualization

- Right column — Real-time graphs, Live video stream, and Mission Control Panel

A dark, high-contrast "mission-control HUD" visual language was used deliberately: a deep space-navy background, a signal-cyan / amber / red status colour system (matching real avionics indicator conventions), a Rajdhani display typeface for headers, and a JetBrains Mono face for all telemetry figures so values read like real instrumentation. The 4-digit error code is rendered as a physical 7-segment LED module, since the fault system is the most CanSat-specific element of the brief and was chosen as the page's signature visual element. The layout is responsive and reflows to a 2-column stack below 1400px width.

# 4. Top Control Bar

Implemented in the sticky <header> of index.html and wired in serial.js / app.js.

- Connect Serial — requests a Web Serial port using navigator.serial.requestPort()

- Start / Stop Telemetry — begins or halts the serial read loop / demo simulator

- Sync PC Time — resets the mission elapsed-time (T+) reference to the current PC time

- Reset Packet — clears telemetry history, graphs, map path and packet counters

- Export CSV / Export Graph — triggers DataManager.exportCsv() and Charts.exportAsImages()

- Demo Sim toggle — enables a built-in telemetry generator so the GCS can be demonstrated without hardware connected

# 5. Mission Control Panel

Mission-critical commands are grouped in missioncontrol.js with a confirm() guard before any command is logged as sent, and a simulated command → acknowledgement transition to mirror real serial command/ack behaviour.

| Command | Code | Feedback Behaviour |
| --- | --- | --- |
| Manual Separation | 0x01 | Confirmation prompt → SENT (amber) → ACK (green) in command log |
| Emergency Parachute Deployment | 0x02 | Confirmation prompt → SENT (amber) → ACK (green) in command log |
| Redundant Activation | 0x03 | Confirmation prompt → SENT (amber) → ACK (green) in command log |

# 6. Telemetry Display

telemetry.js continuously parses comma-separated telemetry lines into a structured packet object, and datamanager.js writes each field into the Container Telemetry and Payload Telemetry panels separately, as required. Container telemetry covers packet count, mission time, altitude, pressure, temperature, battery voltage, descent rate and GPS satellite count. Payload telemetry covers roll, pitch, yaw and GPS latitude / longitude / altitude.

# 7. Error Code System

A 4-digit live fault monitor is implemented in errorcodes.js exactly per the assignment's error-code table, with 0 = no error and 1 = error/fault:

| Digit | Condition | 0 Meaning | 1 Meaning |
| --- | --- | --- | --- |
| 1 | Descent Rate | Within 8–10 m/s | Outside safe range |
| 2 | GPS Availability | GPS data available | GPS data unavailable |
| 3 | Payload Separation | Separated successfully | Separation failure |
| 4 | Emergency Parachute | Parachute inactive | Emergency parachute activated |

Digits update dynamically from live telemetry every packet, and each digit is colour-coded (green = OK, red = fault) in both the 7-segment display and the legend list beneath it, with a glow / pulse treatment on fault for fast operator recognition.

# 8. Real-Time Graphs

charts.js maintains five independent Chart.js line charts — altitude, pressure, temperature, descent rate and battery voltage — each holding a rolling 60-point window and updating with animation disabled for smooth, low-latency real-time rendering.

# 9. Tracking Map

map.js renders a Leaflet.js map on OpenStreetMap tiles, placing a live marker at the current GPS fix and extending a polyline with every new fix to show the full mission trajectory / path history, with the map view panning to follow the payload.

# 10. Orientation Visualization

orientation.js implements a dynamic 3D attitude model in Three.js: a CanSat body mesh with a nose cone and axis helper rotates live on the roll, pitch and yaw telemetry channels, smoothly interpolated frame-to-frame over a reference grid that acts as an artificial horizon.

# 11. Live Video Streaming

video.js uses the MediaDevices getUserMedia() and enumerateDevices() APIs to list available cameras, let the operator choose one, and start/stop a live payload-camera-style video feed with an on-screen LIVE/OFF status indicator.

# 12. Data Management Features

datamanager.js keeps a full in-memory telemetry log for the session and uses the Blob API / File download links to implement:

- CSV export of the complete telemetry log with a timestamped filename

- Graph export — each Chart.js canvas exported as a PNG image

- Packet reset — clears telemetry history, graphs, map path and all live readouts

# 13. Testing Strategy

As required, the microcontroller supplied in the WeGyanik Kit is used as the real-time telemetry streaming device. arduino/dummy_telemetry.ino generates simulated CanSat telemetry (altitude descent, pressure, temperature, battery drain, GPS drift, attitude and fault flags) and streams one CSV packet per second over USB serial at 9600 baud, matching the exact packet format expected by telemetry.js. This was used to test telemetry reception, graph updates, error-code logic, map tracking and orientation visualization under multiple simulated mission conditions (nominal descent, GPS-loss, out-of-range descent rate, and parachute-fault scenarios).

For development and demonstration without hardware connected, a software-only Demo Sim mode (serial.js) reproduces the same packet format and update cadence, so every panel of the GCS can be exercised end-to-end from within the browser alone.

# 14. Technology Stack Summary

| Layer | Tools Used |
| --- | --- |
| Structure & Layout | HTML5, CSS3, CSS Grid, Flexbox |
| Interactivity / State | Vanilla JavaScript (ES6), DOM Manipulation |
| Real-Time Graphs | Chart.js |
| Tracking Map | Leaflet.js + OpenStreetMap tiles |
| Orientation Visualization | Three.js / WebGL |
| Live Video | MediaDevices getUserMedia API |
| Hardware Link | Web Serial API |
| Data Export | Blob API, File download |
| Hardware Testing | Arduino IDE (WeGyanik Kit microcontroller), PySerial-compatible CSV protocol |

# 15. Deliverables Checklist

| Deliverable | Status | Location in Package |
| --- | --- | --- |
| Ground Control Software Source Code | Complete | index.html, css/, js/ |
| Executable / Application Files | Complete | index.html (open directly in a browser) |
| Project Documentation / Report | Complete | docs/CanSat_GCS_Project_Report.docx |
| UI Screenshots | See note below | docs/README.md |
| Demonstration Video | To be recorded by student | docs/README.md |
| Sample Telemetry Logs | Complete | samples/telemetry_log_sample.csv |
| Exported CSV Samples | Complete | samples/exported_telemetry_sample.csv |
| Graph Export Samples | Complete | samples/graph_export_sample.png |
| Hardware Test Sketch | Complete | arduino/dummy_telemetry.ino |

Note on UI Screenshots and the Demonstration Video: both must be captured directly from the running application on the student's own machine (open index.html in Chrome or Edge, optionally connect the WeGyanik Kit or enable Demo Sim), since they need to reflect a live, interactive session. Step-by-step capture instructions are provided in docs/README.md.

# 16. Evaluation Criteria Mapping

| Criteria | Weight | How It Is Met |
| --- | --- | --- |
| UI/UX Design | 15% | Single-page CSS Grid dashboard, consistent HUD colour/type system, responsive layout, colour-coded status indicators |
| Telemetry Handling | 20% | telemetry.js parses every packet field; container/payload data shown separately and updated continuously |
| Real-Time Visualization | 20% | Chart.js graphs, Leaflet map, Three.js orientation model and 7-segment error display all update live per packet |
| Mission Control Features | 15% | Three mission-critical commands with confirmation, SENT→ACK feedback and a persistent command log |
| Graphing and Tracking | 10% | Five rolling real-time graphs; live GPS marker and full path-history polyline |
| Orientation and Video Systems | 10% | Roll/pitch/yaw-driven 3D model; camera-selectable live video with start/stop and status indication |
| Code Quality and Scalability | 10% | Modular per-concern JS files, single telemetry store, documented packet protocol, reusable component pattern |

# 17. Conclusion

The completed CanSat Ground Control Software satisfies every functional section of the assignment brief — interface layout, top control bar, mission control panel, telemetry display, the 4-digit error code system, real-time graphing, GPS tracking, 3D orientation visualization, live video streaming, and full data management — inside a single, modular, mission-oriented dashboard, and has been validated against a dummy telemetry stream generated by the WeGyanik Kit microcontroller as specified in the assignment's testing strategy.